F5 AUTOMATION WITH ANSIBLE

Implementation & Demo Guide

Automated creation of F5 LTM Health Monitor, Pool, Pool Members and HTTPS Virtual Server

| Prepared by | Kailas Pathade |
| --- | --- |
| Mobile | 7777050805 |
| Email | kailasdreams@gmail.com |
| Demo Scope | Ansible Automation Platform / AWX + F5 BIG-IP LTM |

Professional Demo Document

# 1. Demo Objective

This demo shows how Ansible can automate common F5 BIG-IP LTM configuration tasks through the F5 Networks Ansible collection. The workflow creates an HTTPS health monitor, an application pool, two pool members, and an HTTPS virtual server.

# 2. Automation Flow

| Step | Component | Purpose |
| --- | --- | --- |
| 1 | Ansible / AWX Project | Project contains the F5 automation playbook and supporting files. |
| 2 | Inventory | Defines the F5 BIG-IP target used by the automation job. |
| 3 | Credential | Stores the F5 connection credentials used by the job template. |
| 4 | Job Template | Connects the project, inventory and credential into a repeatable job. |
| 5 | Playbook Execution | Creates the monitor, pool, members and virtual server. |
| 6 | Validation | Successful job output confirms the configuration tasks completed. |

# 1. F5 Inventory

Description: This screen shows the Ansible inventory used for the F5 automation project. The inventory provides the target F5 device that the automation job will manage.

# 2. F5 Host Variables

Description: This screen shows the F5 host entry and its connection variables. The host is associated with the F5 management address used by the playbook.

# 3. Ansible Project Configuration

Description: This screen shows the Ansible project configuration. The project points to the automation content and establishes the project directory used by the job.

# 4. F5 Credential

Description: This screen shows the credential configured for the F5 automation. The credential is supplied to the job template so connection details do not need to be hard-coded into the playbook.

# 5. Job Template

Description: This screen shows the job template that combines the F5 project, inventory and credential. The template provides a repeatable way to launch the F5 automation.

# 6. Successful Automation Run

Description: This screen shows the completed automation job. The task output confirms execution of the HTTPS monitor, pool, pool-member and virtual-server tasks.

F5 Evidences:

# 3. F5 LTM Automation Playbook

The following playbook content is retained from the supplied demo document. It uses the f5networks.f5_modules collection and defines the F5 provider from inventory and supplied credentials.

---

- name: Create F5 LTM VIP and Pool

hosts: all

connection: local

gather_facts: false

collections:

- f5networks.f5_modules

vars:

f5_provider:

server: "{{ ansible_host }}"

user: "{{ f5_username }}"

password: "{{ f5_password }}"

validate_certs: false

tasks:

- name: Show F5 connection information

ansible.builtin.debug:

msg:

- "Inventory host: {{ inventory_hostname }}"

- "F5 management IP: {{ ansible_host }}"

- name: Create HTTPS health monitor

f5networks.f5_modules.bigip_monitor_https:

name: MON_APP01

provider: "{{ f5_provider }}"

send: "GET /"

receive: "HTTP"

interval: 5

timeout: 16

state: present

- name: Create F5 pool

f5networks.f5_modules.bigip_pool:

name: POOL_APP01

lb_method: round-robin

monitors:

- MON_APP01

provider: "{{ f5_provider }}"

state: present

- name: Add pool member 1

f5networks.f5_modules.bigip_pool_member:

pool: POOL_APP01

name: APP01_MEMBER_1

host: 10.10.10.11

port: "443"

provider: "{{ f5_provider }}"

state: present

- name: Add pool member 2

f5networks.f5_modules.bigip_pool_member:

pool: POOL_APP01

name: APP01_MEMBER_2

host: 10.10.10.12

port: "443"

provider: "{{ f5_provider }}"

state: present

- name: Create virtual server

f5networks.f5_modules.bigip_virtual_server:

name: VIP_APP01_HTTPS

destination: 192.168.1.200

port: "443"

pool: POOL_APP01

ip_protocol: tcp

profiles:

- http

provider: "{{ f5_provider }}"

state: present

# 4. Configuration Created by the Demo

| Object | Name / Address | Key Setting | Purpose |
| --- | --- | --- | --- |
| HTTPS Health Monitor | MON_APP01 | GET / ; expects HTTP | Checks application availability. |
| Pool | POOL_APP01 | Round-robin | Groups application servers behind the VIP. |
| Pool Member 1 | APP01_MEMBER_1 | 10.10.10.11:443 | Backend application endpoint. |
| Pool Member 2 | APP01_MEMBER_2 | 10.10.10.12:443 | Backend application endpoint. |
| Virtual Server | VIP_APP01_HTTPS | 192.168.1.200:443 | Client-facing HTTPS VIP. |

# 5. Demo Talking Points

Ansible provides a repeatable and structured method to automate F5 configuration.

The F5 provider is built from inventory and runtime credential variables.

The playbook uses F5-specific Ansible modules instead of manually entering tmsh commands.

The automation sequence creates the health monitor first, then the pool, pool members and virtual server.

The successful job output provides an auditable execution trail for the change.

# 6. Key Takeaway

Outcome: The demo demonstrates a complete F5 LTM application onboarding flow through Ansible, from inventory and credentials through job execution and successful creation of the required LTM objects.

Kailas Pathade  |  7777050805  |  [EMAIL ID – Kailasdreams@gmail.com]